- Total renewable freshwater (long-term) — ~35–38 billion m³/yr.

- Per-capita renewable water — ~3,000 m³/person/yr.

- External water dependency — ~70–77 %.

- Mean national precipitation — ~447 mm/yr (1992–2021).

- Rivers (count) — >8,300.

- Reservoirs (count) — ~135.

- Reservoirs total storage — ~21.5 km³.

- Area equipped for irrigation (AQUASTAT) — 1,446 thousand ha.

- Irrigated area (GMIA) — 1,426,000 ha.

- Irrigation from surface water — 1,329,300 ha.

- Irrigation from groundwater — 96,700 ha.

- Total water withdrawal — ~12.8–13.0 billion m³/yr (2017–2021).

- Agriculture share of withdrawals — ~76 %.

- National irrigation efficiency — ~37 %.

- Conveyance losses (national) — ~25–30 %.

- Total irrigation canals (inventory range) — ~47,000–51,755 km.

- Earthen canals share — ~73 %.

- Lined canals share — ~27 %.

- Collector-drainage length — ~29,640–33,015 km.

- Irrigated area with drainage — ~45–50 %.

- Mingachevir storage — 15.73 km³.

- Mingachevir surface area — 605 km².

- Mingachevir length — 70 km.

- Mingachevir width — 18 km.

- Mingachevir max depth — 75 m.

- Shamkir total volume — 2,677 million m³.

- Shamkir exploited volume — 1,425 million m³.

- Shamkir surface area — ~116 km².

- Shamkir HPP capacity — ~380–420 MW.

- Shamkir generation (reported) — ~772 GWh/yr.

- Jeyranbatan volume — 186 million m³.

- Jeyranbatan area — 13.9 km².

- Jeyranbatan length — 8.74 km.

- Jeyranbatan width — 2.15 km.

- Jeyranbatan shoreline — 23.3 km.

- Aras reservoir capacity — ~1.254 km³.

- Taxtakorpu capacity — ~0.270 km³.

- Sarsang capacity — ~560–575 million m³.

- Sarsang surface area — ~14.2 km².

- Samur–Absheron canal length — ~195 km.

- Samur–Absheron avg discharge — ~55 m³/s.

- Upper Shirvan length — ~122 km.

- Upper Shirvan design capacity — ~78 m³/s.

- Upper Shirvan irrigated area — ~100–127 thousand ha.

- Upper Shirvan (rebuild target length) — ~204 km.

- Upper Shirvan (target capacity) — ~180 m³/s.

- Upper Shirvan (target service area) — ~228,000 ha.

- Upper Karabakh canal length — ~172 km.

- Upper Karabakh irrigated area — ~76–100 thousand ha.

- Canal rehab commissioned — 2,277 km (2024–2025).

- Earthen canals absolute length — 39,336 km.

- Lined canals absolute length — 14,569 km.

- Automatic hydrological stations — 40 units.

- Traditional hydrological stations — 44 units.

- Automatic meteorological stations — 51 units.

- Water bodies monitored — 53 units (since 2023).

- Groundwater monitoring sites — 98 units (Upper Kura RBD).

- Irrigation fee — 0.5 AZN per 1,000 m³.

- Household water tariff — 0.70 AZN/m³ (illustrative).

- Water supply cost (Baku) — 1.91 AZN/m³.

- Water connection fee — 200 AZN (one-time).

- Sewer connection fee — 345 AZN (one-time).

- Farmer subsidies paid — 362.5 million AZN (2024).

- Farmers supported — ~380,704 (2024).

- WUAs supported (WB) — 379 units (2011–2018).

- WUA members — ~760,000 people.

- Agriculture workforce share — ~37 % (2023).

- Salt-affected land — ~615,000 ha.

- Kura–Aras non-saline soils — ~495,000 ha.

- Slightly saline soils — ~382,000 ha.

- Saline soils — ~415,000 ha.

- Strongly saline soils — ~449,000 ha.

- Extremely saline soils — ~174,000 ha.

- Okhchuchay Ni exceedance — ~7.1× norm.

- Okhchuchay Fe exceedance — ~4–5.2× norm.

- Okhchuchay Cu–Mo exceedance — ~2× norm.

- Low DO episode — ~4–5 mg/L (vs ~6 mg/L typical).

- Jeyranbatan UF main — 83.5 km.

- Urban sewer coverage (Baku) — ~72–78 %.

- Pump stations (legacy count) — ~900+.

- Electricity tariff (industry/agri) — 0.106 AZN/kWh.

- Sugovushan reservoir volume — 5.86 million m³.

- Sugovushan dam length — ~630 m.

- Sugovushan dam height — ~28 m.

- Sugovushan main canal length — ~5.2 km.

- Sugovushan right-bank capacity — 50 m³/s.

- Sugovushan left-bank capacity — 20 m³/s.

- Sugovushan intended service area — ~96,217 ha.

- Khudafarin reservoir capacity — 1.612 km³.

- Khudafarin HPP capacity — ~102 MW.

- Khudafarin stored (19-May-2024) — ~503 million m³.

- Giz Galasi reservoir total — ~62 million m³.

- Giz Galasi stored (19-May-2024) — ~42 million m³.

- Aras reservoir total — ~1.35 km³.

- Aras surface area — ~145 km².

- Aras HPP capacity — ~44 MW.

- “Great Return” irrigated support (joint) — ~400,000 ha (Aras system).

- New South main canal length — 52 km (Qız Qalası → Cəbrayıl/Füzuli).

- New South main canal capacity — 50 m³/s.

- Automatic weather radars — 2 units (Doppler).

# 1) GIS maps (canals, reservoirs, sluices, pumps, service areas)

- Reservoirs (count) — 153

- Total live storage — 21.9 × 10⁹ m³

- Mingachevir storage — 15,730 × 10⁶ m³

- Shamkir storage — 2,677 × 10⁶ m³

- Aras storage — 1,254 × 10⁶ m³

- Sarsang storage — 565 × 10⁶ m³

- Upper Karabakh Canal length — 172.4 km

- Agstafa Canal length — 72.3 km

- Agstafa annual diversion — 120 × 10⁶ m³/yr

- All irrigation canals (length, inv. A) — 47,058 km

- All irrigation canals (length, inv. B) — 51,755 km

- Mil–Mughan collector–drainage network — 1,277 km

- Mil–Mughan irrigation canals (regional) — 1,268 km

- Irrigated area served (Mil–Mughan) — 120,000 ha

- Mil–Karabakh collector length — 152 km

- Mil–Karabakh collector design discharge — 25 m³/s

# 2) Telemetry (flow, level, gate %, pumps — logging & comms)

- Stage/flow logging interval — 15 min

- Fast alert interval — 5 min

- Routine telemetry send interval — 1–4 h

- Emergency telemetry send — 15 min

- Rain gauge minimum interval — ≤15 min

- LoRa EU868 duty cycle — 1%

- LoRa EU868 max channels — 16

- LoRa EU868 max EIRP — +16 dBm

- LoRa RX sensitivity (SF7, 125 kHz) — −123 dBm

- LoRa RX sensitivity (SF12, 125 kHz) — −137 dBm

- NB-IoT bandwidth — 180 kHz

- NB-IoT maximum coupling loss — 164 dB

# 3) Water quality (EC, pH, turbidity, DO, ions, nutrients, SAR/ESP, algal, oil/TPH, metals)

- EC (no restriction) — <0.7 dS/m

- EC (slight–moderate) — 0.7–3.0 dS/m

- EC (severe) — >3.0 dS/m

- SAR (low hazard) — <3

- SAR (moderate) — 3–9

- SAR (severe) — >9

- Boron guideline (sensitive) — 0.33–0.75 mg/L

- Boron guideline (moderately tolerant) — 0.75–2.0 mg/L

- Boron guideline (tolerant) — 2.0–3.0 mg/L

- Chloride sensor calibration — 10 & 1,000 mg/L

- Nitrate sensor calibration — 1 & 100 mg/L

- DO optical cap life — 15–18 months

- Turbidity zero check — at instrument detection limit (daily)

- Drip clogging risk (Iron, low/mod/high) — <0.2 / 0.2–1.5 / >1.5 mg/L

- Drip clogging risk (Manganese, low/mod/high) — <0.1 / 0.1–1.5 / >1.5 mg/L

- Drip clogging risk (TSS, low/mod/high) — <50 / 50–100 / >100 mg/L

- Drip clogging risk (TDS, low/mod/high) — <500 / 500–2,000 / >2,000 mg/L

- Drip clogging risk (H₂S, low/mod/high) — <0.2 / 0.2–2.0 / >2.0 mg/L

- Drip clogging risk (Bacteria, low/mod/high) — <10,000 / 10,000–50,000 / >50,000 CFU/mL

- Iron staining threshold (overhead) — >0.3 mg/L Fe

- Micro-irrigation iron clogging onset — >0.1 mg/L Fe

- Recommended Fe limit (irrigation water) — 5.0 mg/L

- Recommended Mn limit (irrigation water) — 0.2 mg/L

- Lead limit (irrigation water, guidance) — 5.0 mg/L

- Lithium limit (citrus sensitive) — 0.075 mg/L

- Micro-irrigation chlorination (continuous) — 1–2 mg/L free Cl

- Micro-irrigation chlorination (shock) — 10–20 mg/L free Cl

- pH (low clogging risk) — <7.0

- pH (high clogging risk) — >8.0

- WHO/EPA taste threshold (TDS) — 500 mg/L

- Soil salinity rule of thumb — ECe ≈ 3 × ECw (LF ≥15%)

- Minimum reservoir record fields — 12

- Minimum canal segment fields — 15

- Service polygon mapping scale — 1:25,000

- GPS horizontal accuracy (RMS) — ≤ 3 m

- Max canal centerline vertex spacing — 50 m

- Attribute text field length — 128 characters

- Sensor heartbeat interval — 60 min

- Max telemetry packet size — 256 bytes

- Max daily packets (cap) — 1,440

- Clock drift tolerance — ±2 s/day

- Data gap fill limit — ≤ 120 min

- Uptime SLA — 99.5 %

- Firmware update size limit — 1.5 MB

- RSSI alarm threshold — < −110 dBm

- Battery low threshold — 20 %

- Charge controller absorb setpoint — 14.4 V

- Arsenic limit (irrigation) — ≤ 0.10 mg/L

- Cadmium limit (irrigation) — ≤ 0.01 mg/L

- Chromium (total) limit — ≤ 0.10 mg/L

- Copper limit — ≤ 0.20 mg/L

- Zinc limit — ≤ 2.0 mg/L

- Nickel limit — ≤ 0.20 mg/L

- Selenium limit — ≤ 0.02 mg/L

- Molybdenum limit — ≤ 0.01 mg/L

- Cobalt limit — ≤ 0.05 mg/L

- Vanadium limit — ≤ 0.10 mg/L

- Aluminum limit — ≤ 5.0 mg/L

- Mercury limit — ≤ 0.002 mg/L

- Boron limit (fruit trees) — ≤ 0.5 mg/L

- Chloride (sprinkler leaf burn risk) — > 100 mg/L

- Bicarbonate precipitation risk — > 1.5 meq/L

- Nitrate-N drinking guideline — 10 mg/L

- Total phosphorus bloom trigger — > 0.10 mg/L

- Chlorophyll-a bloom alert — > 20 µg/L

- Microcystin-LR alert — 1 µg/L

- Turbidity caution for drip — > 50 NTU

- UV254 high organics — > 0.10 1/cm

- Rain gauge resolution — 0.2 mm/tip

- Anemometer resolution — 0.1 m/s

- Evaporation pan coefficient (Kp) — 0.70

- Reservoir level measurement precision — 0.01 m

- Reservoir inflow log interval — 24 h

- Incident response time target — ≤ 24 h

- Data retention period — ≥ 5 years

- API rate limit — 60 requests/min

- Alert debounce window — 15 min

- Report generation time — ≤ 60 s

- Wheat seasonal water requirement — 450–650 mm

- Maize seasonal water requirement — 550–800 mm

- Cotton seasonal water requirement — 700–1300 mm

# 4) Weather & inflow (stations, reservoir logs, reanalysis/models)

- ERA5-Land horizontal resolution — ~9 km

- ERA5-Land temporal resolution — hourly

- ERA5-Land period — from 1950

- CHIRPS grid — 0.05° (~5 km)

- CHIRPS period — from 1981

- JRC Global Surface Water resolution — 30 m

- JRC Global Surface Water period — 1984–2021

- ECMWF HRES resolution — ~9 km

- ECMWF ENS resolution — ~9 km

- ECMWF HRES runs per day — 4 (00/06/12/18Z)

# 5) Events & service (calibration, maintenance)

- DO 1-point calibration stabilization — 5–10 min

- ORP sensor life — 12–24 months

- Conductivity sensor dry-air check — <1 µS/cm

- Electrode cleaning strokes — 10–12 passes

- Nitrate span (1 ↔ 100 mg/L) — 90–130 mV

- Chloride span (10 ↔ 1,000 mg/L) — 80–130 mV

- Depth rating (ion-selective cable) — 17 m

- Sensor guard weight (stackable) — ≤5 lb

- pH/DO/EC verification frequency — daily (during use)

- Turbidity verification frequency — daily

# 6) Permits & support (routes, reference sensors, lab; tariffs)

- Water delivery cost — 1.91 AZN/m³

- Household water tariff (Baku/Absheron) — 0.70 AZN/m³

- Household water tariff (other regions) — 0.60 AZN/m³

- Wastewater tariff (population) — 0.30 AZN/m³

- One-time water connection — 200 AZN

- One-time sewer connection — 345 AZN

- Urea export price (FOB avg 2022) — 539 USD/t

- Urea import price (CIF avg 2022) — 787 USD/t

# 7) Agronomy & costs (calendars, soils, ROI inputs)

- Annual river water abstracted (irrigation) — 11,000–13,000 × 10⁶ m³/yr

- System water losses (share) — ~26%

- System water losses (volume example) — ~3,121.4 × 10⁶ m³/yr

- Dam capacity per capita (2001) — 2,628 m³/person

- Dam capacity per capita (2020) — 2,125 m³/person

- Water treatment facilities commissioned (2010–2020) — 45 units

- Added WT capacity (2010–2020) — 1.46 × 10⁶ m³/day

- Project water pipelines built — 479.8 km

- Project sewer pipelines built — 360.8 km

- Project WWTP total capacity (5 cities) — 26,850 m³/day

- Installed water meters (project) — 21,588 units

# 8) Groundwater (status, wells, risks)

- Annual groundwater pumping — ~2.9 × 10⁹ m³/yr

- Groundwater well depths (typical range) — 120–200 m

- Mineral water reserves — ~20,000 m³/day

- Thermal water reserves — ~130,000 m³/day

- High-discharge karst spring example — >100 L/s

- Hydrometric stations measured (liberated areas, campaign) — 10

# 9) Pollution sources & industrial risks

- Sumgayit sewage outfall pipes (legacy) — 17

- Remediation soil removal depth — 0.4 m

- Remediated site area (pilot) — ~2 ha

- Zagatala WWTP capacity — 10,000 m³/day

- Gabala Food Town WWTP upgrade — 2,400 m³/day

- Industrial WWTP (oil & gas case) — 18,000 m³/day

- Drainage diversion (south outfall shift) — ~50 km

- Salinized lands (high estimate) — ~1,000,000 ha

- Salinized lands (other study) — ~600,000 ha

- Collector discharge re-use potential — 25 m³/s

# 10) Crop thresholds (advisory logic)

- Sweet corn salinity threshold (ECe) — 1.7 dS/m

- Sweet corn yield decline per dS/m (above threshold) — 11.5%

- Field corn grain decline per dS/m (ECw > ~5.5 dS/m) — 10.1%

- Sweet corn fresh-ear decline per dS/m (above 5.8 dS/m) — 7.7%

- Sugar beet salinity threshold (ECe) — 5.3 dS/m

- Alfalfa threshold / slope — 2.0 dS/m / 7.3%

- Tall wheatgrass threshold / slope — 7.5 dS/m / 4.2%

- Example leaching calc — ECw 1.2 dS/m; ET 800 mm/season; system efficiency 0.65

# 11) Karabakh & key reservoirs (extra numeric context)

- Khudafarin reservoir capacity — 1.612 km³

- Khudafarin HPP capacity — ~102 MW

- Khudafarin stored (19-May-2024) — ~503 × 10⁶ m³

- Giz Galasi reservoir total — ~62 × 10⁶ m³

- Giz Galasi stored (19-May-2024) — ~42 × 10⁶ m³

- Sarsang HPP capacity — 50 MW

- Sarsang reservoir — 560–575 × 10⁶ m³

- Sugovushan reservoir — 5.86 × 10⁶ m³

- Sugovushan main canal — 5.2 km

- Sugovushan right-bank capacity — 50 m³/s

- Sugovushan left-bank capacity — 20 m³/s

- Sugovushan intended service area — ~96,217 ha

- Aras reservoir total — ~1.35 km³

- Aras surface area — ~145 km²

- Aras HPP capacity — ~44 MW

- New South main canal length — 52 km

- New South main canal capacity — 50 m³/s

- Automatic weather radars — 2 units

# AZERBAIJAN WATER & IRRIGATION — NUMERIC FACTS (COPY-READY)

- Water resources (national balance)

- Total renewable freshwater (long-term average): ~35–38 billion m³/yr. Source: World Bank / FAO AQUASTAT (policy & country notes).

- Per-capita renewable water (illustrative year): ~3,000 m³/person/yr. Source: World Bank (country diagnostics).

- External dependency (share of resources formed outside borders): ~70–77%. Source: World Bank (country diagnostics).

- Mean national precipitation: ~447 mm/yr (1992–2021 norm). Source: UNFCCC National Adaptation Plan (AZ).

- Rivers in Azerbaijan: >8,300 watercourses. Source: national/encyclopedic references.

- Reservoirs in the country: ~135 total; combined storage ~21.5 km³. Source: World Bank (sector notes).

- Irrigation footprint & efficiency

- Area equipped for irrigation (AQUASTAT): 1,446 thousand ha (latest FAO series year). Source: FAO AQUASTAT factsheet (Azerbaijan).

- GMIA mapped irrigated area (historic): 1,426,000 ha (surface water 1,329,300 ha; groundwater 96,700 ha). Source: FAO GMIA country page.

- Agriculture share of water withdrawals: ~76%. Source: World Bank (water accounts).

- Total water withdrawal (recent range): ~12.8–13.0 billion m³/yr (2017–2021). Source: World Bank (water accounts).

- Overall irrigation efficiency (national): ~37%. Source: World Bank (sector diagnostics, 2024).

- Conveyance (transport) losses (national ballpark): ~25–30%. Source: World Bank (operations & sector notes).

- Network lengths, lining, drainage

- Total irrigation canals (different inventories): ~47,000–51,755 km. Source: sector inventories (FAO / academic summaries).

- Earthen vs lined canal share: ~73% earthen; ~27% lined. Source: sector assessments (infrastructure status).

- Collector–drainage network length: ~29,640–33,015 km. Source: sector inventories (FAO / academic summaries).

- Irrigated area covered by subsurface drainage: ~45–50%. Source: World Bank (project docs & diagnostics).

- Typical annual conveyance loss example (Upper Shirvan): ~323 million m³/yr lost (estimate cited in local studies). Source: national/academic summaries.

- Major reservoirs (key figures)

- Mingachevir: storage 15.73 km³; surface area 605 km²; length 70 km; width 18 km; max depth 75 m. Source: encyclopedic/official entries.

- Shamkir: total volume 2,677 million m³; exploited volume 1,425 million m³; surface area ~116 km²; HPP installed capacity ~380–420 MW; reported generation ~772 GWh/yr. Source: plant/reservoir briefs.

- Jeyranbatan: volume 186 million m³; area 13.9 km²; length 8.74 km; width 2.15 km; shoreline 23.3 km. Source: utility/reservoir briefs.

- Araz (Aras) reservoir: capacity ~1.254 km³. Source: regional water resources notes.

- Taxtakorpu reservoir: capacity ~0.270 km³. Source: regional water resources notes.

- Sarsang reservoir: capacity ~560–575 million m³; surface area ~14.2 km². Source: regional project briefs.

- Major canals (length, capacity, service)

- Samur–Absheron main canal: ~195 km; average discharge ~55 m³/s. Source: canal profiles.

- Upper Shirvan: ~122 km; legacy design capacity ~78 m³/s; irrigated area ~100–127 thousand ha. Source: canal profiles & encyclopedic entries.

- Upper Shirvan (reconstruction target): ~204 km; ~180 m³/s; service area ~228,000 ha (plan). Source: government / contractor project notes.

- Upper Karabakh: ~172 km; irrigated area ~76–100 thousand ha (historic range). Source: academic/government summaries.

- Monitoring & telemetry (hydro-met)

- Automatic hydrological stations: ~40 units (recent years). Source: WMO / regional EWS briefs.

- Traditional hydrological stations: ~44 units (recent years). Source: WMO / national hydromet.

- Automatic meteorological stations: ~51 units (grew from 34 to 51 after 2020). Source: WMO / national hydromet.

- Water bodies currently monitored (since 2023): ~53 (rivers, lakes, reservoirs). Source: UNECE/national monitoring updates.

- Water quality & incidents (selected)

- National monitoring (historical scope): ~50 sampling points across ~42 water bodies; lab suite includes EC, pH, temperature, turbidity, DO, major ions (Na, Ca, Mg, K, Cl, SO₄, HCO₃), nutrients (NO₃-N, PO₄-P), SAR/ESP, and trace metals. Source: UNECE/national monitoring manuals.

- Pollution hotspot (Okhchuchay episodes): exceedances approx. Ni ~7×, Fe ~4–5×, Cu/Mo ~2× above norms in 2021 events. Source: ministry/press technical notes.

- Low DO event with fish stress (Mingachevir warm spell): DO ~4–5 mg/L vs ~6 mg/L typical. Source: local press/technical coverage.

- Economics, pricing, subsidies

- Irrigation water state fee: 0.5 AZN per 1,000 m³. Source: World Bank / OECD water economics notes.

- Urban tariff vs supply cost (illustrative): tariff 0.70 AZN/m³ vs cost 1.91 AZN/m³ (Baku). Source: utility/tariff press briefings.

- Connection fees (Baku & regions): water 200 AZN (one-time); sewer 345 AZN (one-time). Source: tariff council press.

- Farmer subsidies paid (2024): ~362.5 million AZN to ~380,704 farmers. Source: ministry/press releases.

- Institutions & users

- Water Users’ Associations (WB-supported cohort): 379 WUAs; ~760,000 members (2011–2018). Source: World Bank project results.

- Agriculture share of workforce: ~37% (2023). Source: national statistics summaries.

- Karabakh focus — Suqovuşan (Tərtərçay) system

- Suqovuşan reservoir volume: 5.86 million m³. Source: regional project/news briefs.

- Dam dimensions: length ~630 m; height ~28 m; crest width ~10 m (as cited). Source: regional project/news briefs.

- Head (main) canal length: ~5.2 km. Source: regional project/news briefs.

- Outlet capacities: right-bank canal 50 m³/s; left-bank canal 20 m³/s. Source: regional project/news briefs.

- Intended downstream service area (historic planning): ~96,217 ha across multiple districts. Source: regional project/news briefs.

- New & planned works (useful for GIS layers)

- Upper Shirvan reconstruction target: ~204 km; ~180 m³/s; ~228,000 ha service; includes Lake Hajigabul restoration effect. Source: government/contractor releases.

- Qız Qalası → Cəbrayıl/Füzuli main canal (South): ~52 km; ~50 m³/s design. Source: regional development news.

- New/rehab reservoirs (examples, liberated areas): Hakarıçay ~91 million m³; Barğuşadçay ~67 million m³ (planning figures). Source: regional development news.

- Extra agronomy/soils context (for AI rules & ROI)

- Salt-affected land (legacy estimate, irrigated plains): ~615,000 ha (severity varies by zone). Source: national/academic summaries.

- Typical method mix: furrow/border dominant; drip/sprinkler still minority share (government promoting expansion). Source: MoA/program notes.

- Subsidy signal for modern irrigation: example support level cited around 400 AZN/ha for modern vs 140 AZN/ha for traditional in recent program briefs. Source: MoA/program notes.

## TEXT FACTS (SHORT, SOURCE-BASED)

- Upper Şirvan rebuild: planned to extend and concrete-line to ~204 km with ~180 m³/s capacity, serving ~228,000 ha and cutting seepage losses. Source: government announcements / contractor project pages.

- Qız Qalası system: a new ~52-km main canal (design ~50 m³/s) from Qız Qalası reservoir is under preparation for Cəbrayıl–Füzuli, backed by joint Iran–Azerbaijan Aras projects. Source: regional development news.

- Suqovuşan: 5.86 mcm storage; 5.2 km head canal; 50/20 m³/s right/left bank canals; dam ~630×28×10 m. Critical to reviving Tərtər-Goranboy-Bərdə irrigation. Source: regional project/news briefs.

- Monitoring network boost: ~40 automatic hydrological and ~51 automatic meteorological stations added after 2020 to improve early warning and water accounting. Source: WMO / regional EWS briefs.

- Hotspot pollution (Okhchuchay): ~5–7× exceedances for Ni/Fe/Cu/Mo in recorded events — a prime target for continuous sensors and farmer alerts in your platform. Source: ministry/press technical notes.

- Losses & efficiency: earthen canals (~73%) drive ~25–30% conveyance losses; overall irrigation efficiency ~37% — your sensor+software can quantify and reduce this. Source: World Bank / sector diagnostics.

## HOW THIS MAPS TO YOUR PILOT DATA INGEST (DIRECT TO YOUR CRITERIA)

- GIS layers (Shapefile/GeoJSON): reservoirs (Mingachevir, Shamkir, Jeyranbatan, Suqovuşan, Sarsang, Taxtakorpu, Qız Qalası); main/secondary canals (Samur–Absheron, Upper Şirvan, Upper Karabakh, Suqovuşan right/left); sluices, pump stations; service polygons (ha) for each scheme.

- Telemetry (5-year history + live): hydrological station series (levels/flows), reservoir inflow/outflow logs; gate opening % and pump on/off (where operators can share). Start with the ~40 hydro + 51 meteo automatic stations; add manual series as CSV.

- Water quality parameters (priority list): EC, pH, temperature, turbidity, DO, Na/Ca/Mg/K, Cl, SO₄, HCO₃, NO₃-N, PO₄-P, SAR/ESP; plus chlorophyll/cyanobacteria, oil/TPH, and heavy metals (As, Pb, Cd, Cr, Cu, Zn, Hg) in hotspot reaches (e.g., Okhchuchay segment).

- Weather & inflows: daily precipitation/temperature from nearest stations; reservoir inflow/outflow journals (daily or monthly).

- Incidents & service: dated records of leaks, contamination, algae blooms, maintenance/weed-clearing, repairs (location + date + short note).

- Permits & ops support: UAV/USV flight permissions; 1–2 reference sensors (calibration); lab chain-of-custody for confirmatory metals/nutrients (where sensor flags exceedances).

- Agronomy & costs (for ROI): crop calendars by region; soil maps (salinity/texture/pH where available); fertilizer prices/subsidies; irrigation fee (0.5 AZN/1,000 m³); energy price for pumps; typical seasonal water volumes by crop (to compute savings).

# Azerbaijan Water & Irrigation — Structured Facts (copy-ready)

## 1) Water resources (national balance)

- Total renewable freshwater (long-term average): ~35–38 billion m³/year.

- Per-capita renewable water (illustrative year): ~3,000 m³/person/year.

- Share of renewable resources formed outside Azerbaijan (external dependency): ~70–77 %.

- Mean national precipitation: ~447 mm/year (1992–2021 norm).

- Rivers in Azerbaijan: >8,300 watercourses.

- Reservoirs in the country: ~135 with ~21.5 km³ total storage.

Sources: national/EEA indicators and syntheses.

## 2) Irrigation footprint and efficiency

- Area equipped for irrigation (AQUASTAT): 1,446 thousand ha (latest FAO series year).

- Area equipped for irrigation (GMIA map figure): 1,426,000 ha; of which surface water 1,329,300 ha, groundwater 96,700 ha.

- National irrigation efficiency (World Bank): ~37 %.

- Conveyance (transport) losses—national ballpark: ~25–30 %.

- Agriculture share of total water withdrawals: ~76 % (recent years).

- Total water withdrawal (recent range): ~12.8–13.0 billion m³/year.

Sources: FAO AQUASTAT/GMIA; WB/EEA water-use indicators.

## 3) Network lengths, lining, drainage

- Total irrigation canals (different inventories): ~47,000–51,755 km.

- Earthen vs lined canals: ~73 % earthen, ~27 % lined.

- Collector–drainage network length: ~29,640–33,015 km.

- Irrigated area covered by subsurface drainage: ~45–50 %.

Sources: regional water assessments and inventories.

## 4) Major reservoirs (key figures)

- Mingachevir: storage 15.73 km³; surface area 605 km²; length 70 km; width 18 km; max depth 75 m.

- Shamkir: total volume 2,677 million m³; exploited volume 1,425 million m³; surface area ~116 km²; HPP installed capacity ~380–420 MW; reported generation ~772 GWh/year.

- Jeyranbatan: volume 186 million m³; area 13.9 km²; reservoir length 8.74 km; width 2.15 km; shoreline 23.3 km.

- Araz (Aras) Dam reservoir: capacity ~1.254 km³.

- Taxtakorpu: capacity ~0.270 km³.

- Sarsang: capacity ~560–575 million m³; surface area 14.2 km².

Sources: encyclopedic/official entries.

## 5) Major canals (length, capacity, service)

- Samur–Absheron main canal: ~195 km; average discharge ~55 m³/s.

- Upper Shirvan: ~122 km; legacy design capacity ~78 m³/s; irrigated area ~100–127 thousand ha.

- Upper Shirvan (reconstruction target): length ~204 km; capacity ~180 m³/s; service area ~228,000 ha (plan).

- Upper Karabakh canal: length ~172 km; irrigated area ~76–100 thousand ha (historic range).

Sources: canal project briefs and references.

## 6) Monitoring & telemetry (hydro-met)

- Automatic hydrological stations: ~40 active units (recent years).

- Traditional hydrological stations: ~44 (recent years).

- Automatic meteorological stations: ~51 (grew from 34 to 51).

- Water bodies currently monitored (since 2023): ~53 rivers/lakes/reservoirs.

Sources: UNFCCC/UNECE monitoring updates.

## 7) Water quality & incidents (selected)

- National monitoring (historical scope): ~50 sampling points across ~42 water bodies; lab suite includes major ions, nutrients, metals, etc.

- Pollution hotspot (Okhchuchay episodes): Ni ~7×, Fe ~4–5×, Cu/Mo ~2× above norms in events reported 2021.

- Low DO event & fish stress (Mingachevir, warm spell): DO ~4–5 mg/L vs typical ~6 mg/L.

Sources: ministry/press technical notes.

## 8) Economics, pricing, subsidies

- Irrigation water state fee: 0.5 AZN per 1,000 m³.

- Urban tariff vs supply cost (illustrative): 0.70 AZN/m³ tariff vs 1.91 AZN/m³ cost in Baku.

- Connection fees (Baku & regions): water 200 AZN (one-time); sewer 345 AZN (one-time).

- Farmer subsidies paid (2024): ~362.5 million AZN to ~380,704 farmers.

Sources: tariff council, reports, subsidy portals.

## 9) Institutions & users

- Water Users’ Associations (WB-supported cohort): 379 WUAs serving ~760,000 members (2011–2018).

- Agriculture share of workforce (context): ~37 % (2023).

Sources: World Bank results; trade stats.

## 10) Karabakh focus — Suqovuşan (Tərtərçay) system

- Reservoir volume: 5.86 million m³.

- Dam dimensions: length ~630 m, height ~28 m (crest width ~10 m noted in some briefs).

- Head (main) canal length: ~5.2 km.

- Outlet capacities: Right-bank 50 m³/s, Left-bank 20 m³/s.

- Intended downstream service area (historic planning): ~96,217 ha across multiple districts.

Sources: project/regional reports.

## 11) New & planned works (illustrative, useful for GIS layer planning)

- Upper Shirvan reconstruction: target ~204 km, ~180 m³/s, ~228,000 ha service; lake Hajigabul restoration effect cited.

- Qız Qalası → Cəbrayıl/Füzuli main canal (South): ~52 km, ~50 m³/s design.

- New/rehab reservoirs (examples): Hakarıçay ~91 million m³, Barğuşadçay ~67 million m³ (planning figures).

## 12) What to ingest for your pilot (direct mapping to your criteria)

- GIS layers: reservoirs, main/secondary canals (Samur–Absheron, Upper Shirvan, Upper Karabakh, Suqovuşan outlets), sluices, pump stations, service polygons; export as Shapefile/GeoJSON.

- Telemetry (5-year history + live): hydrological station series (levels/flows), reservoir release logs; where possible add gate opening % and pump on/off from operators (CSV/API). Counts above show autos (40 hydro / 51 meteo) you can target first.

- Water quality (parameters): EC, pH, T°, turbidity, DO, Na/Ca/Mg/K, Cl, SO₄, HCO₃, NO₃-N, PO₄-P, SAR/ESP; plus chlorophyll/cyanobacteria, oil/TPH, heavy metals (As, Pb, Cd, Cr, Cu, Zn, Hg) in hotspots (e.g., Okhchuchay).

- Weather & inflows: station precipitation/temperature; reservoir inflow/outflow journals (daily/monthly).

- Incidents & service: records of leaks, contamination, algae blooms, maintenance/weed-clearing, repairs (date + location). (Use press/agency logs where available.)

- Permits & ops support: UAV/USV flight permissions; reference sensors for calibration; lab chain-of-custody for confirmatory metals/nutrients. (Regulatory overviews exist for UAV ops.)

- Agronomy & costs (for ROI): crop calendars by region, soil maps (salinity, texture), fertilizer prices/subsidy rules, irrigation water fee, energy cost, typical field application volumes by crop.